YAPP(tm) protocol for Packet Radio binary file transfer

(c) 1986, Jeff Jacobsen - WA7MBL

Revision 1.1   06/23/86

----------------------------------------------------------

YAPP PACKET TYPES

Size of packet data ranges from 2-258 bytes.

len = length of following bytes (not including len)

RR Rcv_Rdy      ACK  01

RF Rcv_File     ACK  02

AF Ack_EOF      ACK  03   [1.1]

AT Ack_EOT      ACK  04   [1.1]

SI Send_Init    ENQ  01

HD Send_Hdr     SOH  len  (Filename)  NUL  (File Size in ASCII)  NUL  (Opt)

DT Send_Data    STX  len  (Data)    {if len=0 then data length = 256}

EF Send_EOF     ETX  01

ET Send_EOT     EOT  01

NR Not_Rdy      NAK  len  (Optional Reason in ASCII)

CN Cancel       CAN  len  (Optional Reason in ASCII)

CA Can_Ack      ACK  05

{Following are for use in Server mode}

RI Rcv_Init     ENQ   02   len   (Filespec requested - wildcard allowed)

TX Text         DLE   len  (ASCII text for display)  {to send text from server}

{Unimplement - Reserved for Server Commands}

UU Comands      ENQ   03

ENQ   04

ENQ   05

...

ENQ   FF

STATE TABLES for SEND

State         Send Pkt        Upon                  Next State

S (Send)      SI              RR                    SH

RF                    SD

NR                    Start/Done

First few timeouts    S

RI                    S

Other/Timeout         Abort

SH (Hdr)      HD              RF                    SD

NR                    Start/Done

Other/Timeout         Abort

SD (Data)     DT              (Not EOF)             SD

(EOF)                 SE

SE (EOF)      EF              AF (More files)       SH

AF (No More Files)    ST

Other/Timeout         Abort

ST (EOT)      ET              AT                    Start/Done

Other/Timeout         Start/Done

STATE TABLES for RECEIVE

State      On    If           Do          Send Pkt       Next State

R (Rec)    SI (ready)                       RR              RH

SI (opt)*       open file        RF              RD

SI (not ready)                   NR              Start/Done

NR                                               Start/Done

Other/Timeout                                    Abort

(opt)*  If header info not desired it is ok to

open file and indiate ready for file

RH (Hdr)   HD              open file        RF              RD

HD (no room)                     NR              Start/Done

SI                                               RH

ET                               AT              Start/Done

Other/Timeout                                    Abort

RD (Data)  DT              store data                       RD

EF              close file       AF              RH

Other/Timeout                                    Abort

OTHER STATE

Any State          On CN         Send CA                   Start/Done

except CW

CW (Can_Wait)      On CA                                   Start/Done

On CN         Send CA                   CW

Timeout       Display Err Msg           Start/Done

Abort                            Send CN                   CW

--------

Starting States

For 2 users not using Remote Server

Sender goes to State S, Receiver to State R after manually

exchanging file names.  Since File Name is known, Recevier

may optionally send RF from state R and bypass file header.

Sender should give Receiver ample time to get into Receive

State.

For Remote Server Mode

USER (Start)                                       Next State

User wishing to Send                               S

User wishing to Receive          Send RI           R

Other Server Request             Send COMMAND      Start

SERVER (Start)

Gets SI                                            R

Gets RI             (file fnd)                     S

(no file)    Send NR           Start

Gets COMMAND                    (Do Command)       Start

---------------------------------------------------------

All states except Send Data should timeout if nothing is heard

for a period of time Tc (Crash timer).  Send Data isn't expecting

any input and if it should receive some it must be a Cancel, or

a TX (text) packet, otherwise Abort.

--------

Revision 1.1 - Added AF & AT to acknowledge End of File and

End of Transfer.  This allows an automated sender to know that

a file was received successfully.  It also helps get past a

problem in the TNC-2 code by requiring the sender to wait for the

Ack-EOT before switching from transparent mode back to converse

mode.